Energy saving through counter

Present project is designed using AT89S52 microcontroller to count the number of person present in the room and to save the energy used by the appliances. The project utilizes the IL74 optocoupler which is used to detect the visitors. If a person enters in the room then optocoupler will detect the person and the buzzer will generate a sound and there will be an increment in the number of person on the LCD. LCD used in our project is JHD162A which is used to display the number of person. If a person leaves the room then optocoupler will detect the person and buzzer will generate a sound and there will be a decrement in the number of person. On the basis of number of person, different appliances like bulbs, fans, A.C., T.V. etc. can be switched on/off automatically by using relays. A relay is an electrically controllable switch. It allows the isolation of two separate sections of a system with two different voltage sources. Relay used in our project is electromagnetic relay. When there will be no person in the room then no device will work. Devices will work only on basis of no. of person present in the room. Thus, this project not only counts the no. of person, it also saves the energy used by the appliances.

CIRCUIT DIAGRAM:

HARDWARE USED:

| SR.NO | COMPONENT NAME | COMPONENT DESCRIPTION |
| --- | --- | --- |
| 1. | IC 7805,7812 | Voltage regulator IC |
| 2. | IC AT89S52 | Microcontroller |
| 3. | Transformer 9-0-9 V | Step Down Transformer |
| 4. | Switches | Push to on switches |
| 5. | Resistors | Current limiter |
| 6. | Capacitors | As a Filter |
| 7. | LED | Visual Display |
| 8. | LCD | For no. of visitors display |
| 9. | Buzzer | Audio Indication |
| 10. | Quartz Crystal | For providing clock frequency |
| 11. | Diodes | As a Rectifier |
| 12. | Connecting Wires | For Desired Connections |
| 13. | General purpose PCB | For mounting the components |
| 14. | Transistor | Used as a switch for buzzer |
| 15. | Potentiometer | To vary the contrast of LCD |
| 16. | Optocoupler | To detect the visitor |
| 17. | ULN-2004 | To connect relays |
| 18. | Relays | To connect the appliances |
| 19. | Bulb | To show the output |

SOFTWARE USED: