Performance of a Speculative Transmission Scheme for Scheduling-Latency Reduction

Abstract:

This work was motivated by the need to achieve low latency in an input-queued centrally-scheduled cell switch for high-performance computing applications; specifically, the aim is to reduce the control-path latency incurred between issuance of a request and arrival of the corresponding grant. The minimum latency in switches with centralized scheduling comprises two components, namely, the control-path latency and the data-path latency, which in a practical high-capacity, distributed switch implementation can be far greater than the cell duration. We introduce a speculative transmission scheme to significantly reduce the average control-path latency by allowing cells to proceed without waiting for a grant, under certain conditions. It operates in conjunction with any centralized matching algorithm to achieve a high maximum utilization and incorporates a reliable delivery mechanism to deal with failed speculations. An analytical model is presented to investigate the efficiency of the speculative transmission scheme employed in a non-blocking N*NR input-queued crossbar switch with receivers R per output. Using this model, performance measures such as the mean delay and the rate of successful speculative transmissions are derived. The results demonstrate that the control-path latency can be almost entirely eliminated for loads up to 50%. Our simulations confirm the analytical results.

Existing System:

- Brikoff-von-newmann Switch which is eliminate the scheduler. It incurs a worst-case latency penalty of N time slots. It has to wait for exactly N time slots for the next opportunity.

- Control and data path-latencies comprise serialization and de-serialization delays, propagation delay, processing delay between request and response.

Limitation of Existing System

- It incurs a worst-case latency penalty of N time slots. It has to wait for exactly N time slots for the next opportunity.

- Control and data path-latencies comprise serialization and de-serialization delays, propagation delay, processing delay between request and response.

Proposed System:

- We propose a novel method to combine speculative and scheduled transmission in a cross bar switch.

- Speculative modes of operation reduced latency at low utilization.

- Scheduled modes of operation achieve high maximum throughput.

Advantage:

- The speculative transmission that does not have to wait for grant hence low latency.

- The scheduled transmission achieve high maximum throughput.

System Requirements:

Hardware:

PROCESSOR         :  PENTIUM IV 2.6 GHz

RAM		      :	512 MB DD RAM

MONITOR	      :	15” COLOR

HARD DISK           :	20 GB

CDDRIVE	       :	LG 52X

KEYBOARD           :	STANDARD 102 KEYS

MOUSE	       :	3 BUTTONS

Software:

Front End              :  Java, Swing

Tools Used             :  JFrameBuilder

Operating System   :  WindowsXP

Back End                : SQL Server 2000