A REPORT 
ON

“DESIGN & FABRICATION OF SUSPENSION SYSTEMS”

SUBMITTED TO 

TOWARDS PARTIAL FULFILLMENT OF THE DEGREE OF

BACHELOR OF ENGINEERING
IN
MECHANICAL ENGINEERING

DEPARTMENT OF MECHANICAL ENGINEERING

THE SUSPENSION SYSTEM

The job of a car suspension is to maximize the friction between the tires and the road surface, to provide steering stability with good handling and to ensure the comfort of the passengers. In this article, we'll explore how car suspensions work, how they've evolved over the years and where the design of suspensions is headed in the future.

If a road were perfectly flat, with no irregularities, suspensions wouldn't be necessary. But roads are far from flat. Even freshly paved highways have subtle imperfections that can interact with the wheels of a car. It's these imperfections that apply forces to the wheels. According to Newton's laws of motion, all forces have both magnitude and direction. A bump in the road causes the wheel to move up and down perpendicular to the road surface. The magnitude, of course, depends on whether the wheel is striking a giant bump or a tiny speck. Either way, the car wheel experiences a vertical acceleration as it passes over an imperfection.

Without an intervening structure, all of wheel's vertical energy is transferred to the frame, which moves in the same direction. In such a situation, the wheels can lose contact with the road completely. Then, under the downward force of gravity, the wheels can slam back into the road surface. What you need is a system that will absorb the energy of the vertically accelerated wheel, allowing the frame and body to ride undisturbed while the wheels follow bumps in the road.The study of the forces at work on a moving car is called vehicle dynamics, and you need to understand some of these concepts in order to appreciate why a suspension is necessary in the first place. Most automobile engineers consider the dynamics of a moving car from two perspectives:

- Ride - a car's ability to smooth out a bumpy road

- Handling - a car's ability to safely accelerate, brake and corner

These two characteristics can be further described in three important principles - road isolation, road holding and cornering. The table below describes these principles and how engineers attempt to solve the challenges unique to each.

| Principle | Definition | Goal | Solution |
| --- | --- | --- | --- |
| Road Isolation | The vehicle's ability to absorb or isolate road shock from the passenger compartment | Allow the vehicle body to ride undisturbed while traveling over rough roads. | Absorb energy from road bumps and dissipate it without causing undue oscillation in the vehicle. |
| Road Holding | The degree to which a car maintains contact with the road surface in various types of directional changes and in a straight line (Example: The weight of a car will shift from the rear tires to the front tires during braking. Because the nose of the car dips toward the road, this type of motion is known as "dive." The opposite effect -- "squat" -- occurs during acceleration, which shifts the weight of the car from the front tires to the back.) | Keep the tires in contact with the ground, because it is the friction between the tires and the road that affects a vehicle's ability to steer, brake and accelerate. | Minimize the transfer of vehicle weight from side to side and front to back, as this transfer of weight reduces the tire's grip on the road. |
| Cornering | The ability of a vehicle to travel a curved path | Minimize body roll, which occurs as centrifugal force pushes outward on a car's center of gravity while cornering, raising one side of the vehicle and lowering the opposite side. | Transfer the weight of the car during cornering from the high side of the vehicle to the low side. |

- Car Suspension Parts

The suspension of a car is actually part of the chassis, which comprises all of the important systems located beneath the car's body.

These systems include:

The frame - structural, load-carrying component that supports the car's engine and body, which are in turn supported by the suspension

- The suspension system - setup that supports weight, absorbs and dampens shock and helps maintain tire contact

- The steering system - mechanism that enables the driver to guide and direct the vehicle

- The tires and wheels - components that make vehicle motion possible by way of grip and/or friction with the road

So the suspension is just one of the major systems in any vehicle.

Springs: Sprung and Unsprung Mass
The sprung mass is the mass of the vehicle supported on the springs, while the unsprung mass is loosely defined as the mass between the road and the suspension springs. The stiffness of the springs affects how the sprung mass responds while the car is being driven.Tightly sprung cars, such as sports cars are less forgiving on bumpy roads, but they minimize body motion well, which means they can be driven aggressively,even around corners.

Dampers: Shock Absorbers

Unless a dampening structure is present, a car spring will extend and release the energy it absorbs from a bump at an uncontrolled rate. The spring will continue to bounce at its natural frequency until all of the energy originally put into it is used up. A suspension built on springs alone would make for an extremely bouncy ride and, depending on the terrain, an uncontrollable car.

Enter the shock absorber, or snubber, a device that controls unwanted spring motion through a process known as dampening. Shock absorbers slow down and reduce the magnitude of vibratory motions by turning the kinetic energy of suspension movement into heat energy that can be dissipated through hydraulic fluid. To understand how this works, it's best to look inside a shock absorber to see its structure and function.

A shock absorber is basically an oil pump placed between the frame of the car and the wheels. The upper mount of the shock connects to the frame (i.e., the sprung weight), while the lower mount connects to the axle, near the wheel (i.e., the unsprung weight). In a twin-tube design, one of the most common types of shock absorbers, the upper mount is connected to a piston rod, which in turn is connected to a piston, which in turn sits in a tube filled with hydraulic fluid. The inner tube is known as the pressure tube, and the outer tube is known as the reserve tube. The reserve tube stores excess hydraulic fluid.

When the car wheel encounters a bump in the road and causes the spring to coil and uncoil, the energy of the spring is transferred to the shock absorber through the upper mount, down through the piston rod and into the piston. Orifices perforate the piston and allow fluid to leak through as the piston moves up and down in the pressure tube. Because the orifices are relatively tiny, only a small amount of fluid, under great pressure, passes through. This slows down the piston, which in turn slows down the spring.

Shock absorbers work in two cycles -- the compression cycle and the extension cycle. The compression cycle occurs as the piston moves downward, compressing the hydraulic fluid in the chamber below the piston. The extension cycle occurs as the piston moves toward the top of the pressure tube, compressing the fluid in the chamber above the piston. A typical car or light truck will have more resistance during its extension cycle than its compression cycle. With that in mind, the compression cycle controls the motion of the vehicle's unsprung weight, while extension controls the heavier, sprung weight.

All modern shock absorbers are velocity-sensitive -- the faster the suspension moves, the more resistance the shock absorber provides. This enables shocks to adjust to road conditions and to control all of the unwanted motions that can occur in a moving vehicle, including bounce, sway, brake dive and acceleration squat.

- Dampers: Struts and Anti-sway Bars

Common strut design

Another common dampening structure is the strut -- basically a shock absorber mounted inside a coil spring. Struts perform two jobs: They provide a dampening function like shock absorbers, and they provide structural support for the vehicle suspension. That means struts deliver a bit more than shock absorbers, which don't support vehicle weight -- they only control the speed at which weight is transferred in a car, not the weight itself.

Because shocks and struts have so much to do with the handling of a car, they can be considered critical safety features. Worn shocks and struts can allow excessive vehicle-weight transfer from side to side and front to back. This reduces the tire's ability to grip the road, as well as handling and braking performance.

Photo courtesy HSW Shopper
Anti-sway bars

Anti-sway Bars
Anti-sway bars (also known as anti-roll bars) are used along with shock absorbers or struts to give a moving automobile additional stability. An anti-sway bar is a metal rod that spans the entire axle and effectively joins each side of the suspension together.

When the suspension at one wheel moves up and down, the anti-sway bar transfers movement to the other wheel. This creates a more level ride and reduces vehicle sway. In particular, it combats the roll of a car on its suspension as it corners. For this reason, almost all cars today are fitted with anti-sway bars as standard equipment, although if they're not, kits make it easy to install the bars at any time.

- Suspension Types: Front

So far, our discussions have focused on how springs and dampers function on any given wheel. But the four wheels of a car work together in two independent systems -- the two wheels connected by the front axle and the two wheels connected by the rear axle. That means that a car can and usually does have a different type of suspension on the front and back. Much is determined by whether a rigid axle binds the wheels or if the wheels are permitted to move independently. The former arrangement is known as a dependent system, while the latter arrangement is known as an independent system. In the following sections, we'll look at some of the common types of front and back suspensions typically used on mainstream cars.

Dependent Front Suspensions 
Dependent front suspensions have a rigid front axle that connects the front wheels. Basically, this looks like a solid bar under the front of the car, kept in place by leaf springs and shock absorbers. Common on trucks, dependent front suspensions haven't been used in mainstream cars for years.

Independent Front Suspensions 
In this setup, the front wheels are allowed to move independently. The MacPherson strut, developed by Earle S. MacPherson of General Motors in 1947, is the most widely used front suspension system, especially in cars of European origin.

The MacPherson strut combines a shock absorber and a coil spring into a single unit. This provides a more compact and lighter suspension system that can be used for front-wheel drive vehicles.

The double-wishbone suspension, also known as an A-arm suspension, is another common type of front independent suspension.

While there are several different possible configurations, this design typically uses two wishbone-shaped arms to locate the wheel. Each wishbone, which has two mounting positions to the frame and one at the wheel, bears a shock absorber and a coil spring to absorb vibrations. Double-wishbone suspensions allow for more control over the camber angle of the wheel, which describes the degree to which the wheels tilt in and out. They also help minimize roll or sway and provide for a more consistent steering feel. Because of these characteristics, the double-wishbone suspension is common on the front wheels of larger cars.

- Suspension Types: Rear

Historical Suspensions Sixteenth-century wagons and carriages tried to solve the problem of "feeling every bump in the road" by slinging the carriage body from leather straps attached to four posts of a chassis that looked like an upturned table. Because the carriage body was suspended from the chassis, the system came to be known as a "suspension" -- a term still used today to describe the entire class of solutions. The slung-body suspension was not a true springing system, but it did enable the body and the wheels of the carriage to move independently. Semi-elliptical spring designs, also known as cart springs, quickly replaced the leather-strap suspension. Popular on wagons, buggies and carriages, the semi-elliptical springs were often used on both the front and rear axles. They did, however, tend to allow forward and backward sway and had a high center of gravity. By the time powered vehicles hit the road, other, more efficient springing systems were being developed to smooth out rides for passengers.

Dependent Rear Suspensions 
 If a solid axle connects the rear wheels of a car, then the suspension is usually quite simple -- based either on a leaf spring or a coil spring. In the former design, the leaf springs clamp directly to the drive axle. The ends of the leaf springs attach directly to the frame, and the shock absorber is attached at the clamp that holds the spring to the axle. For many years, American car manufacturers preferred this design because of its simplicity.

The same basic design can be achieved with coil springs replacing the leaves. In this case, the spring and shock absorber can be mounted as a single unit or as separate components. When they're separate, the springs can be much smaller, which reduces the amount of space the suspension takes up.

Independent Rear Suspensions 
If both the front and back suspensions are independent, then all of the wheels are mounted and sprung individually, resulting in what car advertisements tout as "four-wheel independent suspension." Any suspension that can be used on the front of the car can be used on the rear, and versions of the front independent systems described in the previous section can be found on the rear axles. Of course, in the rear of the car, the steering rack -- the assembly that includes the pinion gear wheel and enables the wheels to turn from side to side -- is absent. This means that rear independent suspensions can be simplified versions of front ones, although the basic principles remain the same.